# Design and Implementation of an FPGA-based Laser Motion Controller

ENGR 433 Project Report	December 19, 2025

Andrei Vasilev	Andrei.Vasilev@wallawalla.edu

Joseph Nguyen            Joseph.Nguyen@wallawalla.edu

GitHub Repository:	https://github.com/nguyjo/pico2-ice-Joystick

Table of Contents

Table of Contents									2

Introduction										3

Project Objectives									4

System Architecture									4

Hardware Components								6

Sensor Principles									7

Software Implementation								8

FPGA Implementation									9

Testing and Results									10

Conclusion										11

Appendix A: C &  SystemVerilog Code							12

LaserDiodeBringUp.sv								12

GyroscopeBringUp.c								13

ServoMotorBringUp.c								17

ServoMotorBringUp.sv								19

main.c										21

top.sv										25

spi_slave.sv									27

Introduction

Modern embedded control systems increasingly rely on the integration of multiple processing elements to meet competing requirements such as computational flexibility, real-time responsiveness, and system reliability. Applications in robotics, aerospace, industrial automation, and human–machine interfaces often require continuous acquisition of sensor data, real-time signal processing, and precise control of actuators. Designing a single processing unit that efficiently satisfies all these requirements is challenging, particularly when strict timing constraints are involved.

Microcontrollers are widely used in embedded systems due to their low cost, ease of programming, and extensive peripheral support. They are well suited for tasks such as sensor interfacing, data processing, and control logic implementation. However, microcontroller-based systems rely on sequential execution and interrupt-driven scheduling, which can introduce timing jitters and limit deterministic behavior when controlling time-critical actuators such as servo motors.

Field-programmable gate arrays (FPGAs), in contrast, offer deterministic timing, parallel execution, and precise control over hardware-level signal generation. These characteristics make FPGAs particularly suitable for real-time tasks such as pulse-width modulation (PWM), high-speed signal processing, and safety-critical control functions. The primary disadvantage of FPGAs is their increased design complexity and reduced flexibility for algorithmic development compared to software-based solutions.

To address these limitations, modern embedded systems often adopt heterogeneous architecture, combining a microcontroller and an FPGA within a single control system. In such architectures, the microcontroller handles high-level tasks such as sensor acquisition, calibration, and computational algorithms, while the FPGA is responsible for deterministic, time-critical signal generation and control. This separation of responsibilities improves system reliability, scalability, and real-time performance.

Project Objectives

The objective of this project is to design and implement a distributed embedded control system that integrates a microcontroller and an FPGA to convert inertial sensor measurements into precise real-time actuator control. The system aims to demonstrate effective partitioning of high-level sensor acquisition, calibration, and orientation estimation tasks on a microcontroller, while delegating timing-critical servo motor control to hardware logic implemented on an FPGA. Through this architecture, the project seeks to achieve deterministic actuator behavior, reliable inter-processor communication, and stable system performance independent of software execution variability. Additionally, the project aims to validate the proposed design through a two-axis servo-driven laser mechanism that provides an intuitive and visual demonstration of the system’s responsiveness and control accuracy.

System Architecture

The proposed system architecture is based on a distributed control approach that separates sensor processing and decision-making from time-critical actuator control. The system consists of four primary functional blocks: an inertial measurement unit (IMU), a microcontroller, an FPGA, and a two-axis servo-driven actuator mechanism. Each block performs a well-defined role, and data flows unidirectionally from sensing to actuation, ensuring predictable and deterministic system behavior.

At the sensing layer, the IMU measures linear acceleration and angular velocity along three orthogonal axes. These raw measurements are transmitted to the microcontroller via an I²C interface. The microcontroller serves as the high-level processing unit, responsible for configuring the IMU, performing sensor calibration, and converting raw sensor data into meaningful orientation information. Specifically, roll and pitch angles are estimated using accelerometer-based tilt calculations, with gyroscope data available for motion detection and bias compensation. All numerical processing and control logic related to orientation estimation are implemented in software, allowing flexibility and ease of modification.

Once the orientation parameters are computed, the microcontroller transmits the processed control information to the FPGA through a dedicated digital interface. This communication represents a clear boundary between the software and hardware domains of the system. By transmitting high-level control commands rather than raw sensor data, the FPGA is relieved from complex arithmetic operations and can focus exclusively on real-time signal generation. This design choice reduces FPGA logic complexity while preserving deterministic behavior.

The FPGA forms the real-time control layer of the system. It receives orientation commands from the microcontroller and converts them into pulse-width modulation signals suitable for driving standard servo motors. The PWM signals are generated entirely in hardware with a fixed frame rate and precise timing resolution, ensuring consistent servo behavior regardless of variations in microcontroller execution timing or communication latency. Two independent PWM channels are implemented to control roll and pitch servos separately, enabling two-axis actuation.

At the actuation layer, two servo motors are arranged in an orthogonal configuration to form a simple gimbal mechanism. The first servo provides horizontal (roll) motion, while the second servo, mounted on top of the first, provides vertical (pitch) motion. A laser module attached to the top servo provides a visual indication of system output, allowing the relationship between IMU orientation and actuator response to be easily observed and evaluated.

Overall, this architecture demonstrates a clear separation of responsibilities between sensing, computation, communication, and actuation. By combining the flexibility of a microcontroller with the deterministic timing capabilities of an FPGA, the system achieves reliable real-time performance while remaining modular and extensible. This architectural approach reflects common design practices used in professional embedded control systems, robotics platforms, and aerospace applications.

Hardware Components

The associated hardware includes the pico2-ice board, which has an RP2350 microcontroller and a Lattice ice40 FPGA onboard. This development board was used during the ENGR 433 course, and was sufficient for the requirements of the Laser Motion Controller.

The LSM6DSOX IMU on the “Adafruit LSM6DSOX + LIS3MDL - Precision 9 DoF IMU” breakout board was chosen to measure and calculate angular coordinateese. Although our chosen IMU has I2C and SPI capabilities, the breakout board only interfaced its I2C interface. To interface the IMU with the RP2350, the I2C support in the pico-sdk repository was utilized.

For the servo motors, that physically created our laser motion controller, 1 servo motor was placed on its side on above the second servo motor, to enable both roll and pitch axes of motion. and a laser module. The “TowerPro SG-5010 – 5010” standard servo motors were used due to its ease of access and its simple qualifications. Because our servo motors did not expect large torque loads, almost any hobby servo motor could be used. These motors were powered by two 5 V batteries, because their current draw needed is too much for the RP2350 or the ice40.

To integrate the laser diode into the servo motors, hot glue was used to attach it to the top servo motor, while both servo motors were also glued together. As for turning the servo motor on using an FPGA pin, an npn transistor circuit with one of the 5 V batteries was used to draw enough current through the diode. A JAN2N1711 npn transistor was used in the circuit. Specifically, its base was connected to the FPGA pin, allowing the laser diode to be switched on and off at the command of our SystemVerilog code, its collector was connected to the laser diode and the 5V battery, and its emitter was connected to ground. To safely drive the laser diode, a current-limiting resistor of 500 Ω resistor was used between the battery, and the laser diode.

Sensor Principles

The system utilizes an inertial measurement unit (IMU) to sense the orientation and motion of the control device. An IMU typically combines multiple sensing elements, most commonly an accelerometer, a gyroscope, and, in some cases, a magnetometer. Each sensor measures a different physical quantity and provides complementary information about the system’s motion and orientation. Understanding the operating principles of these sensors is essential for interpreting the measured data and designing a reliable control system.

The accelerometer measures linear acceleration along three orthogonal axes. In addition to acceleration caused by motion, the accelerometer continuously measures the acceleration due to gravity. When the IMU is stationary, the measured acceleration vector corresponds primarily to the gravity vector, which can be used to estimate the device’s orientation relative to the Earth. By analyzing how the gravity vector is projected onto the sensor axes, roll and pitch angles can be computed. This property makes accelerometers particularly well suited for static and low-frequency orientation estimation. However, accelerometer measurements are sensitive to vibrations and dynamic motion, which can introduce noise and transient errors during rapid movement.

The gyroscope measures angular velocity about each axis, providing information about how fast the device is rotating. Unlike the accelerometer, the gyroscope is insensitive to linear acceleration and gravity, making it highly effective for detecting rapid rotational motion. Angular positions can be estimated by integrating the gyroscope’s angular velocity over time. However, gyroscope measurements are subject to bias and drift, meaning that small measurement errors accumulate over time and lead to increasing orientation error if not corrected. For this reason, gyroscope data is typically combined with accelerometer measurements or used for short-term motion detection rather than long-term absolute orientation estimation.

The magnetometer measures the local magnetic field, which is primarily influenced by the Earth’s magnetic field. By measuring the magnetic field vector, the magnetometer provides an absolute reference for orientation relative to the Earth, enabling estimation of yaw, or heading. While magnetometers are useful for full three-dimensional orientation estimation, they are sensitive to magnetic disturbances caused by nearby ferromagnetic materials and electrical currents. As a result, magnetometer data often requires careful calibration and filtering to produce reliable results.

In this project, the accelerometer and gyroscope are used as the primary sensing elements. The accelerometer provides a stable reference for roll and pitch estimation based on gravity, while the gyroscope supports motion detection and bias calibration. The magnetometer, although available in the IMU hardware, is not utilized because the application does not require yaw estimation. This design choice reduces system complexity while providing sufficient accuracy and responsiveness for the intended joystick-like control application.

Software Implementation

The software component of the system is implemented on a Raspberry Pi Pico microcontroller and is responsible for sensor interfacing, data processing, and high-level control logic. The primary role of the microcontroller is to acquire inertial measurement data from the IMU, perform calibration and orientation estimation, and transmit processed control commands to the FPGA. All software tasks are designed to operate independently of the time-critical actuator control, which is delegated to the FPGA.

The software initializes communication with the inertial measurement unit using the I²C protocol. During system startup, the microcontroller configures the IMU by setting appropriate operating modes, output data rates, and measurement ranges for the accelerometer and gyroscope. This initialization ensures consistent sensor behavior and predictable data sampling throughout system operation. Proper configuration of the I²C interface and sensor registers is essential for reliable data acquisition.

Following initialization, a calibration procedure is performed to compensate for gyroscope bias. During this phase, the IMU is kept stationary for a short period while multiple gyroscope samples are collected and averaged. The resulting bias values are stored and subtracted from subsequent gyroscope measurements. This calibration step reduces long-term drift and improves the stability of motion-related measurements, particularly during slow or subtle movements of the control device.

Once calibration is complete, the software enters its main execution loop. In this loop, raw accelerometer and gyroscope data are continuously read from the IMU. Accelerometer measurements are converted into physical units and used to estimate roll and pitch angles based on the direction of the gravity vector. These calculations provide an absolute orientation reference that remains stable over time, making them suitable for joystick-style control. Gyroscope data is used to detect rotational motion and support dynamic behavior. Although advanced sensor fusion techniques such as Kalman filtering are intentionally omitted to reduce computational complexity, a simple complementary filter is used between accelerometer and gyroscope data.

The computed orientation angles are scaled, constrained to predefined limits, and formatted in 16-bit SPI for transmission to the FPGA. By transmitting high-level orientation commands rather than raw sensor data, the microcontroller reduces communication bandwidth requirements and simplifies the FPGA logic. This design choice reinforces the separation of responsibilities between software-based computation and hardware-based execution.

Between the RP2350 and the ice40, our 16-bit SPI packet allots 8 bits to roll (bottom servo motor), and 8 bits to pitch (top servo motor). During each iteration of the main while loop, the function map_tilt_to_servo calculates a value between 0-180 to be sent over SPI, for both roll and pitch.

Communication between the microcontroller and the FPGA is implemented using a synchronous digital interface. The microcontroller periodically sends updated control values at a fixed rate, ensuring that the FPGA receives timely and consistent command data. The software operates in a continuous loop without strict real-time constraints, relying on the FPGA to maintain deterministic actuator timing.

FPGA Implementation

The FPGA component of the system is responsible for all timing-critical control functions, specifically the generation of precise pulse-width modulation (PWM) signals for servo motor actuation. An iCE40 FPGA is used to implement deterministic hardware logic that operates independently of the microcontroller’s software execution timing. By offloading real-time control tasks to the FPGA, the system achieves consistent and reliable actuator behavior that is not affected by software latency, interruptions, or communication delays.

The FPGA design is implemented using SystemVerilog, a hardware description language (HDL) and operates from a fixed system clock of 48 MHz, specified by main.c. A dedicated timing module divides the system clock to 24 MHz to limit space usage. When producing PWM signals for the 2 servo motors, the algorithm chosen calculates the number of clock ticks the signal should be high, and the number of ticks it should be low, during each PWM cycle. This cycle corresponds to a standard servo update frequency of 50 Hz, with a period of approximately 20 milliseconds. If these clock ticks were based on a 48 MHz clock, then a larger variable would have to be allotted to the clock tick counters.

Control commands received from the microcontroller represent desired orientation angles for the roll and pitch axes. These commands are converted within the FPGA into corresponding PWM pulse widths using a linear mapping function. The mapping process constrains pulse widths to predefined minimum and maximum values to protect the servos from mechanical overtravel. Two independent PWM channels are implemented, allowing simultaneous and parallel control of the roll and pitch servos without resource contention.

To ensure robust operation, the FPGA logic maintains the most recent valid control command and continuously applies it to the PWM generators until a new command is received. This behavior prevents unintended actuator movement in the event of temporary communication interruptions. Because the PWM generation is implemented entirely in hardware, the output timing is fully deterministic and immune to software-induced jitter.

The FPGA design emphasizes simplicity, reliability, and real-time performance. Rather than implementing complex signal processing or control algorithms in hardware, the FPGA focuses exclusively on precise execution of control signals. This design philosophy minimizes logic complexity while maximizing timing accuracy, making the system scalable to additional actuators or more advanced control features in future iterations.

Testing and Results

The system was tested incrementally to verify correct operation of each functional subsystem prior to full system integration. This staged testing approach helped isolate potential issues related to sensor communication, signal timing, and actuator control, and ensured reliable overall system performance.

Initial testing focused on validating communication between the microcontroller and the inertial measurement unit. Successful I²C communication was confirmed by reading raw accelerometer and gyroscope data and observing stable, repeatable measurements when the IMU was held stationary. This was completed using the Serial Monitor Visual Studio Code (VSCode) extension. Sensor configuration registers were verified to ensure correct output data rates and measurement ranges. Gyroscope calibration was tested by monitoring sensor output before and after bias compensation, which resulted in significantly reduced offset and improved measurement stability.

Subsequent tests evaluated the orientation estimation implemented on the microcontroller. Roll and pitch angles were monitored while manually rotating the IMU around each axis. When the IMU was placed flat on a horizontal surface, the computed orientation angles remained close to zero, confirming correct gravity-based alignment. Controlled tilting of the IMU produced proportional changes in the calculated angles, demonstrating that the orientation estimation was both responsive and stable for the intended joystick-style input. To comply with servo motor safety practices, we tested different PWM pulse widths to determine our maximum and minimum widths. The minimum width was 0.6 ms, and the maximum width was 2.4 ms. This corresponded to ~170 degrees of motion in both the roll and pitch servo motors. This exceeded the laser projection requirements of our application, but demonstrated correct and safe use of servo motors.

To test the laser diode, a script was made to blink the laser diode on and off every 1 second. This verified our laser diode transistor circuit, and performed a proper bring-up of the circuit through a SystemVerilog script.

Full system integration testing combined the IMU, microcontroller, FPGA, and servo actuators. During these tests, the IMU was used as a handheld controller, and its orientation directly influenced the position of the two-axis servo mechanism. Tilting the IMU around the roll axis resulted in corresponding horizontal movement of the laser, while tilting around the pitch axis produced vertical movement. The system responded smoothly and consistently to user input, with no unintended oscillations or delays observed during normal operation.

Conclusion

This project successfully demonstrated the design and implementation of a distributed embedded control system that integrates a microcontroller and an FPGA to achieve reliable real-time actuator control based on inertial sensor input. By separating high-level sensor processing and orientation estimation from timing-critical actuator control, the system effectively leveraged the complementary strengths of software-based and hardware-based processing.

The Raspberry Pi Pico microcontroller provided a flexible platform for IMU interfacing, calibration, and orientation computation, while the iCE40 FPGA ensured deterministic generation of servo control signals with precise timing. This architectural partitioning eliminated actuator jitter and improved overall system stability compared to a purely software-based approach. The use of an IMU as a joystick-like input device enabled intuitive two-axis control, and the laser-based visual feedback allowed clear evaluation of system behavior.

Incremental testing and full system integration confirmed that the proposed design met its objectives, producing smooth and responsive servo motion with reliable startup and stable operation. The project highlights the practical benefits of heterogeneous microcontroller–FPGA co-design and illustrates key principles relevant to modern embedded control systems, including modularity, determinism, and scalability.

Overall, the results demonstrate that combining a microcontroller for sensor processing with an FPGA for real-time control provides a robust and extensible solution for embedded control applications. The system architecture and implementation developed in this project form a solid foundation for future enhancements, such as advanced sensor fusion, additional control axes, or closed-loop feedback mechanisms.

Appendix A: C &  SystemVerilog Code

LaserDiodeBringUp.sv

////////////////////////////////////////////////////////////////////////////////

// This SystemVerilog File serves as a bring-up and verification of the Laser Diode

// Transistor Circuit

////////////////////////////////////////////////////////////////////////////////

module LaserDiodeBringUp (

input  logic CLK,    // 48 MHz clock input

output logic LED_G,  // Blue LED output (active-low)

output logic ICE_46  // Laser Diode output (active-low)

);

// Clock and blink frequency parameters

localparam int CLOCK_FREQUENCY = 48_000_000; // 48 MHz

localparam int BLINK_FREQUENCY = 1;          // 1 Hz

// Counter limit for half-period

localparam int COUNTER_MAX = CLOCK_FREQUENCY / (2 * BLINK_FREQUENCY);

// Counter register

logic [$clog2(COUNTER_MAX)-1:0] counter = '0;

// LED toggle logic

always_ff @(posedge CLK) begin

if (counter == COUNTER_MAX - 1) begin

counter <= '0;

LED_G  <= ~LED_G;   // Toggle LED

ICE_46 <= ~ICE_46; // Toggle Laser Diode

end else begin

counter <= counter + 1;

end

end

// Initialize output

initial LED_G = 1'b1; // LED off initially (active-low)

initial ICE_46 = 1'b0; // Laser Diode off initially (active-high)

endmodule

GyroscopeBringUp.c

////////////////////////////////////////////////////////////////////////////////

// This C File serves as a bring-up and verification of the Gyroscope. Functionality was

// verified by seeing gyroscope output through the Serial Monitor VSCode extension.

////////////////////////////////////////////////////////////////////////////////

#include <stdio.h>

#include <math.h>

#include "pico/stdlib.h"

#include "hardware/i2c.h"

// -------------------------------

// LSM6DSOX I2C + registers

// -------------------------------

#define LSM6_ADDR     0x6A

#define CTRL1_XL      0x10

#define CTRL2_G       0x11

#define CTRL3_C       0x12

#define OUTX_L_G      0x22

#define OUTX_L_A      0x28

// Sensitivities (approx, matches your MicroPython constants)

#define ACC_LSB_TO_G        (0.000061f)  // 2g range ~0.061 mg/LSB

#define GYRO_LSB_TO_DPS     (0.07f)      // 2000 dps range ~70 mdps/LSB

static inline float rad2deg(float r) { return r * (180.0f / (float)M_PI); }

// -------------------------------

// I2C helpers

// -------------------------------

static bool lsm6_write_u8(i2c_inst_t *i2c, uint8_t reg, uint8_t val) {

uint8_t buf[2] = { reg, val };

return i2c_write_blocking(i2c, LSM6_ADDR, buf, 2, false) == 2;

}

static bool lsm6_read(i2c_inst_t *i2c, uint8_t reg, uint8_t *dst, size_t n) {

// Write register address, then read

if (i2c_write_blocking(i2c, LSM6_ADDR, &reg, 1, true) != 1) return false;

return i2c_read_blocking(i2c, LSM6_ADDR, dst, n, false) == (int)n;

}

static int16_t s16(uint8_t lo, uint8_t hi) {

return (int16_t)((hi << 8) | lo);

}

static bool lsm6_read3_s16(i2c_inst_t *i2c, uint8_t reg0, int16_t *x, int16_t *y, int16_t *z) {

uint8_t d[6];

if (!lsm6_read(i2c, reg0, d, 6)) return false;

*x = s16(d[0], d[1]);

*y = s16(d[2], d[3]);

*z = s16(d[4], d[5]);

return true;

}

// -------------------------------

// Main

// -------------------------------

int main() {

stdio_init_all();

sleep_ms(1500); // give USB serial time

// I2C1 on GP2(SDA) and GP3(SCL)

i2c_inst_t *I2C = i2c1;

i2c_init(I2C, 10 * 1000); // 10 kHz like your MicroPython

gpio_set_function(2, GPIO_FUNC_I2C);

gpio_set_function(3, GPIO_FUNC_I2C);

gpio_pull_up(2);

gpio_pull_up(3);

printf("\nLSM6DSOX roll/pitch + gyro test (Pico SDK C)\n");

// -------------------------------

// Init sensor

// -------------------------------

// Reset

lsm6_write_u8(I2C, CTRL3_C, 0x01);

sleep_ms(50);

// IF_INC=1 (bit2), BDU=1 (bit6)

lsm6_write_u8(I2C, CTRL3_C, (1u << 2) | (1u << 6));

sleep_ms(10);

// Gyro: ODR=104 Hz, FS=2000 dps

lsm6_write_u8(I2C, CTRL2_G, 0x4C);

sleep_ms(10);

// Accel: ODR=104 Hz, FS=2g

lsm6_write_u8(I2C, CTRL1_XL, 0x40);

sleep_ms(50);

// -------------------------------

// Calibration: gyro bias (keep still)

// -------------------------------

printf("Calibrating gyro bias... keep IMU still on the table (~3s)\n");

const int samples = 60;

int32_t sum_gx = 0, sum_gy = 0, sum_gz = 0;

for (int i = 0; i < samples; i++) {

int16_t gx, gy, gz;

if (!lsm6_read3_s16(I2C, OUTX_L_G, &gx, &gy, &gz)) {

printf("I2C read failed during calibration!\n");

sleep_ms(200);

continue;

}

sum_gx += gx;

sum_gy += gy;

sum_gz += gz;

sleep_ms(50);

}

float bias_gx = (float)sum_gx / (float)samples;

float bias_gy = (float)sum_gy / (float)samples;

float bias_gz = (float)sum_gz / (float)samples;

printf("Gyro bias (raw): %.2f %.2f %.2f\n", bias_gx, bias_gy, bias_gz);

printf("Printing roll/pitch degrees every 0.2s\n\n");

// -------------------------------

// Loop

// -------------------------------

while (true) {

int16_t ax_raw, ay_raw, az_raw;

int16_t gx_raw, gy_raw, gz_raw;

if (!lsm6_read3_s16(I2C, OUTX_L_A, &ax_raw, &ay_raw, &az_raw) ||

!lsm6_read3_s16(I2C, OUTX_L_G, &gx_raw, &gy_raw, &gz_raw)) {

printf("I2C read failed!\n");

sleep_ms(200);

continue;

}

// Convert accel to g

float ax = (float)ax_raw * ACC_LSB_TO_G;

float ay = (float)ay_raw * ACC_LSB_TO_G;

float az = (float)az_raw * ACC_LSB_TO_G;

// Compute roll/pitch from gravity

float roll  = rad2deg(atan2f(ay, az));

float pitch = rad2deg(atan2f(-ax, sqrtf(ay*ay + az*az)));

// Gyro deg/s with bias removed

float gx_dps = ((float)gx_raw - bias_gx) * GYRO_LSB_TO_DPS;

float gy_dps = ((float)gy_raw - bias_gy) * GYRO_LSB_TO_DPS;

float gz_dps = ((float)gz_raw - bias_gz) * GYRO_LSB_TO_DPS;

printf("roll_deg: %7.2f  pitch_deg: %7.2f   |   gyro_dps: %7.2f %7.2f %7.2f\n",

roll, pitch, gx_dps, gy_dps, gz_dps);

sleep_ms(200);

}

}

ServoMotorBringUp.c

////////////////////////////////////////////////////////////////////////////////

// This C File serves as a bring-up and verification for the two servo motors.

// Functionality was verified by moving the motors 180 degrees back and forth every 1

// second

////////////////////////////////////////////////////////////////////////////////

#include "pico/stdlib.h"

#include "hardware/spi.h"

#include "hardware/gpio.h"

#include "ice_cram.h"

#include "ice_fpga.h"

// --- CONFIGURATION ---

#define FPGA_CS_PIN 25      // Manual Chip Select

#define FPGA_CLK_FREQ 48    // We Init at 48MHz, Verilog handles the divider

// Servo Limits (Calculated for 24MHz effective speed)

// 0.6ms = 14400 ticks

// 2.4ms = 57600 ticks

#define SERVO_MIN 14400

#define SERVO_MAX 57600

uint8_t bitstream[] = {

#include "bitstream.h"

};

int main(void) {

stdio_init_all();

// 1. Initialize FPGA

ice_fpga_init(FPGA_DATA, FPGA_CLK_FREQ);

ice_fpga_start(FPGA_DATA);

ice_cram_open(FPGA_DATA);

ice_cram_write(bitstream, sizeof(bitstream));

ice_cram_close();

// 2. Initialize SPI (1 MHz)

spi_init(spi1, 1000 * 1000);

gpio_set_function(24, GPIO_FUNC_SPI); // MISO

gpio_set_function(27, GPIO_FUNC_SPI); // MOSI

gpio_set_function(26, GPIO_FUNC_SPI); // SCK

// Manual Chip Select

gpio_init(FPGA_CS_PIN);

gpio_set_dir(FPGA_CS_PIN, GPIO_OUT);

gpio_put(FPGA_CS_PIN, 1);

spi_set_format(spi1, 8, SPI_CPOL_0, SPI_CPHA_0, SPI_MSB_FIRST);

while (true) {

// --- MOVE TO 0 DEGREES (MIN) ---

uint16_t s1 = SERVO_MIN;

uint16_t s2 = SERVO_MIN;

// Pack 32 bits: [S1 High] [S1 Low] [S2 High] [S2 Low]

uint8_t tx_buf[4];

tx_buf[0] = (s1 >> 8) & 0xFF;

tx_buf[1] = (s1 & 0xFF);

tx_buf[2] = (s2 >> 8) & 0xFF;

tx_buf[3] = (s2 & 0xFF);

gpio_put(FPGA_CS_PIN, 0); // CS Low

spi_write_blocking(spi1, tx_buf, 4); // Send 4 bytes

gpio_put(FPGA_CS_PIN, 1); // CS High

sleep_ms(1000);

// --- MOVE TO 180 DEGREES (MAX) ---

s1 = SERVO_MAX;

s2 = SERVO_MAX;

tx_buf[0] = (s1 >> 8) & 0xFF;

tx_buf[1] = (s1 & 0xFF);

tx_buf[2] = (s2 >> 8) & 0xFF;

tx_buf[3] = (s2 & 0xFF);

gpio_put(FPGA_CS_PIN, 0);

spi_write_blocking(spi1, tx_buf, 4);

gpio_put(FPGA_CS_PIN, 1);

sleep_ms(1000);

}

}

ServoMotorBringUp.sv

////////////////////////////////////////////////////////////////////////////////

// This SystemVerilog File serves as a bring-up and verification for the two servo motors.

// Functionality was verified by moving the motors 180 degrees back and forth every 1

// second

////////////////////////////////////////////////////////////////////////////////

module ServoMotorBringUp (

input  logic CLK,      // 48 MHz Clock

input  logic sck,

input  logic mosi,

input  logic cs_n,

output logic miso,

output logic ICE_48,

output logic ICE_46

);

// --- SPI INTERFACE ---

logic [31:0] spi_data;

logic rx_valid;

// Registers to hold pulse widths

logic [15:0] width_1; // Servo 1 (High 16 bits)

logic [15:0] width_2; // Servo 2 (Low 16 bits)

spi_slave u_spi (

.CLK(CLK),

.sck(sck), .mosi(mosi), .cs_n(cs_n), .miso(miso),

.data_out(spi_data),

.rx_valid(rx_valid)

);

// Latch data when valid packet arrives

always_ff @(posedge CLK) begin

if (rx_valid) begin

width_1 <= spi_data[31:16];

width_2 <= spi_data[15:0];

end

end

// --- 24 MHz PRESCALER ---

// The FPGA runs at 48MHz. We toggle a bit to create a "tick" every 2 cycles.

// This makes our math effective 24MHz.

logic tick_24mhz;

logic [1:0] div_cnt;

always_ff @(posedge CLK) begin

div_cnt <= div_cnt + 1;

tick_24mhz <= (div_cnt[0] == 1'b1);

end

// --- PWM GENERATOR ---

// 20ms period @ 24MHz = 480,000 ticks

logic [18:0] pwm_cnt;

always_ff @(posedge CLK) begin

if (tick_24mhz) begin

if (pwm_cnt >= 479999)

pwm_cnt <= 0;

else

pwm_cnt <= pwm_cnt + 1;

end

end

// Output Logic

assign ICE_48 = (pwm_cnt < width_1) ? 1'b1 : 1'b0;

assign ICE_46 = (pwm_cnt < width_2) ? 1'b1 : 1'b0;

endmodule

main.c

////////////////////////////////////////////////////////////////////////////////

// This C File in the main program sends IMU data over SPI to the ice40.

////////////////////////////////////////////////////////////////////////////////

#include <stdio.h>

#include <math.h>

#include "pico/stdlib.h"

#include "hardware/i2c.h"

#include "hardware/spi.h"

#include "hardware/gpio.h"

#include "ice_cram.h"

#include "ice_fpga.h"

// -------------------- FPGA CONFIG --------------------

#define FPGA_SPI       spi1

#define PIN_MISO       24

#define PIN_CS         25

#define PIN_SCK        26

#define PIN_MOSI       27

#define FPGA_CLK_FREQ  48 // MHz

// -------------------- IMU (LSM6DSOX) --------------------

#define LSM6_ADDR      0x6A

#define CTRL1_XL       0x10

#define CTRL2_G        0x11

#define CTRL3_C        0x12

#define OUTX_L_G       0x22

#define OUTX_L_A       0x28

// --- CRITICAL FIX: Pins 2 and 3 are I2C1 ---

#define IMU_I2C        i2c1

#define IMU_SDA_PIN    2

#define IMU_SCL_PIN    3

// Bitstream

uint8_t bitstream[] = {

#include "bitstream.h"

};

// --- Helpers ---

static void i2c_write_u8(uint8_t reg, uint8_t val) {

uint8_t buf[2] = { reg, val };

i2c_write_blocking(IMU_I2C, LSM6_ADDR, buf, 2, false);

}

static void read3_imu(uint8_t reg0, int16_t *x, int16_t *y, int16_t *z) {

uint8_t buffer[6];

i2c_write_blocking(IMU_I2C, LSM6_ADDR, &reg0, 1, true);

i2c_read_blocking(IMU_I2C, LSM6_ADDR, buffer, 6, false);

*x = (int16_t)((buffer[1] << 8) | buffer[0]);

*y = (int16_t)((buffer[3] << 8) | buffer[2]);

*z = (int16_t)((buffer[5] << 8) | buffer[4]);

}

uint8_t map_tilt_to_servo(float tilt) {

float min_in = -45.0f;

float max_in = 45.0f;

if (tilt < min_in) tilt = min_in;

if (tilt > max_in) tilt = max_in;

return (uint8_t)((tilt - min_in) * 180.0f / (max_in - min_in)); // this number between 0-180 is sent over SPI to ice40

}

int main() {

stdio_init_all();

sleep_ms(100);

// 1. Initialize FPGA

ice_fpga_init(FPGA_DATA, FPGA_CLK_FREQ);

ice_fpga_start(FPGA_DATA);

ice_cram_open(FPGA_DATA);

ice_cram_write(bitstream, sizeof(bitstream));

ice_cram_close();

// 2. Init I2C

i2c_init(IMU_I2C, 400 * 1000);

gpio_set_function(IMU_SDA_PIN, GPIO_FUNC_I2C);

gpio_set_function(IMU_SCL_PIN, GPIO_FUNC_I2C);

gpio_pull_up(IMU_SDA_PIN);

gpio_pull_up(IMU_SCL_PIN);

// 3. Init SPI

spi_init(FPGA_SPI, 1000 * 1000);

gpio_set_function(PIN_MISO, GPIO_FUNC_SPI);

gpio_set_function(PIN_SCK,  GPIO_FUNC_SPI);

gpio_set_function(PIN_MOSI, GPIO_FUNC_SPI);

gpio_init(PIN_CS);

gpio_set_dir(PIN_CS, GPIO_OUT);

gpio_put(PIN_CS, 1);

spi_set_format(FPGA_SPI, 8, SPI_CPOL_0, SPI_CPHA_0, SPI_MSB_FIRST);

// 4. Init IMU

i2c_write_u8(CTRL3_C, 0x01); // Reset

sleep_ms(50);

i2c_write_u8(CTRL3_C, 0x44); // Block Data Update

i2c_write_u8(CTRL2_G, 0x4C); // Gyro

i2c_write_u8(CTRL1_XL, 0x4A); // Accel

sleep_ms(50);

// Initial angle values

float roll = 0.0f;

float pitch = 0.0f;

while (true) {

int16_t axr, ayr, azr;

int16_t gxr, gyr, gzr;

// Read Sensors

read3_imu(OUTX_L_A, &axr, &ayr, &azr);

read3_imu(OUTX_L_G, &gxr, &gyr, &gzr);

// Calculate Angles

float accel_roll  = atan2f((float)ayr, (float)azr) * 180.0f / (float)M_PI;

float accel_pitch = atan2f(-(float)axr, sqrtf((float)ayr*(float)ayr + (float)azr*(float)azr)) * 180.0f / (float)M_PI;

// Complementary Filter

roll  = 0.9f * roll  + 0.1f * accel_roll;

pitch = 0.9f * pitch + 0.1f * accel_pitch;

// --- MAP TO FPGA PROTOCOL ---

// Get the base 0-180 value

uint8_t raw_x = map_tilt_to_servo(roll);

uint8_t raw_y = map_tilt_to_servo(pitch);

// --- INVERT LOGIC HERE ---

// If turning Right makes it go Left, we do (180 - value) to flip it.

// If one axis is correct and the other is wrong, only do math on the wrong one.

uint8_t tx_buf[2];

// Invert X (Roll)

tx_buf[0] = 180 - raw_x;

// Invert Y (Pitch)

tx_buf[1] = 180 - raw_y;

gpio_put(PIN_CS, 0);

spi_write_blocking(FPGA_SPI, tx_buf, 2);

gpio_put(PIN_CS, 1);

sleep_ms(20);

}

}

top.sv

////////////////////////////////////////////////////////////////////////////////

// This SystemVerilog File in the main handles PWM calculation and signal generation. It

// reads data over SPI when the master pulls the chip select low.

////////////////////////////////////////////////////////////////////////////////

module top (

input  logic CLK,    // 48 MHz

input  logic sck, mosi, cs_n,

output logic miso,

output logic ICE_46, // SERVO X

output logic ICE_48, // SERVO Y

output logic ICE_47  // LASER

);

// SPI Interface

logic [15:0] spi_data;

// We ignore rx_valid from slave, we use CS_N logic instead

logic unused_valid;

spi_slave u_spi (

.CLK(CLK), .sck(sck), .mosi(mosi), .cs_n(cs_n), .miso(miso),

.data_out(spi_data), .rx_valid(unused_valid)

);

// --- ROBUST SPI LATCHING ---

// We sample CS_N using the fast system clock (48MHz).

// When CS_N goes from Low (0) to High (1), we grab the data.

logic [1:0] cs_sync;

logic [7:0] angle_x;

logic [7:0] angle_y;

always_ff @(posedge CLK) begin

cs_sync <= {cs_sync[0], cs_n}; // Shift in CS state

// If CS transitions from 0 to 1 (01 binary)

if (cs_sync == 2'b01) begin

angle_x <= spi_data[15:8];

angle_y <= spi_data[7:0];

end

end

// --- MATH & PWM ---

logic [15:0] width_x;

logic [15:0] width_y;

always_ff @(posedge CLK) begin

width_x <= 16'd14400 + (angle_x * 16'd240); // 0.6 ms minimum pulse width

width_y <= 16'd14400 + (angle_y * 16'd240); // 2.4 ms maximum pulse width

end

// 24 MHz Prescaler

logic tick_24mhz;

logic clk_div;

always_ff @(posedge CLK) begin

clk_div <= ~clk_div;

tick_24mhz <= (clk_div == 0);

end

// PWM Counter

logic [18:0] pwm_cnt;

always_ff @(posedge CLK) begin

if (tick_24mhz) begin

if (pwm_cnt >= 479999) pwm_cnt <= 0;

else pwm_cnt <= pwm_cnt + 1;

end

end

assign ICE_46 = (pwm_cnt < width_x);

assign ICE_48 = (pwm_cnt < width_y);

assign ICE_47 = 1'b1; // Laser Always On

endmodule

spi_slave.sv

////////////////////////////////////////////////////////////////////////////////

// This SystemVerilog file in the main program handles and receives the 16-bit SPI packets

// sent from the master. It acts as a FIFO buffer, shifting in bits one at a time. Top.sv reads this FIFO buffer when the chip select is held low.

////////////////////////////////////////////////////////////////////////////////

module spi_slave (

input  logic CLK,

input  logic sck,

input  logic cs_n,

input  logic mosi,

output logic miso,

output logic [15:0] data_out,

output logic rx_valid

);

logic [15:0] shift_reg;

logic [4:0]  bit_cnt;

always_ff @(posedge sck or posedge cs_n) begin

if (cs_n) begin

bit_cnt   <= 5'd0;

rx_valid  <= 1'b0;

end else begin

// Implements SPI Shift Register (Buffer)

shift_reg <= {shift_reg[14:0], mosi}; // Concatenate new MOSI bit into LSB in shift register Buffer

bit_cnt   <= bit_cnt + 1;

if (bit_cnt == 5'd15) begin

data_out <= {shift_reg[14:0], mosi};

rx_valid <= 1'b1;

bit_cnt  <= 5'd0;

end else begin

rx_valid <= 1'b0;

end

end

end

assign miso = shift_reg[15];

endmodule